ASCON+

Scalable Multi-Node ASCON-Secured IoT Communication
A Gateway-Based Architecture with Dynamic Key Exchange

Project Progress Report
October 01, 2026

# Table of Contents

1.  Project Overview

2.  Completed Work Summary

3.  Repository Structure

4.  Environment Setup (setup_pi.sh)

5.  Cryptographic Handshake Implementation (test_crypto.py)

6.  Phase 1 — Ephemeral X25519 ECDH Key Exchange

7.  Phase 2 — HKDF-SHA256 Key Derivation

8.  Phase 3 — ASCON-128 AEAD Encrypt / Decrypt

9.  Security Considerations Addressed

10.  Licensing

11.  What Remains / Next Steps

# 1. Project Overview

ASCON+ is an IoT security project that implements a complete cryptographic communication pipeline designed for resource-constrained devices, specifically targeting the Raspberry Pi Zero W (ARMv6, 512 MB RAM). The project combines two modern cryptographic primitives:

Elliptic Curve Diffie-Hellman (ECDH) key exchange using X25519 for secure, ephemeral session key establishment.

ASCON-128 Authenticated Encryption with Associated Data (AEAD) — a NIST-standardised lightweight cipher — for encrypting sensor data payloads.

The architecture follows a gateway-based model with dynamic key exchange, enabling scalable, multi-node IoT communication where each session derives fresh cryptographic keys that are never reused.

# 2. Completed Work Summary

| # | Component | Status |
| --- | --- | --- |
| 1 | Project repository initialisation (GitHub, README, LICENSE) | ✅ Complete |
| 2 | Raspberry Pi Zero W environment setup script (setup_pi.sh) | ✅ Complete |
| 3 | Ephemeral X25519 ECDH key exchange (Phase 1) | ✅ Complete |
| 4 | HKDF-SHA256 key derivation to 16-byte ASCON key (Phase 2) | ✅ Complete |
| 5 | ASCON-128 AEAD encryption & decryption (Phase 3) | ✅ Complete |
| 6 | Tamper detection / authentication tag verification | ✅ Complete |
| 7 | End-to-end cryptographic handshake simulation | ✅ Complete |

# 3. Repository Structure

The current repository contains the following files:

| File | Size | Purpose |
| --- | --- | --- |
| README.md | 118 B | Project title and one-line description |
| LICENSE | ~35 KB | GNU General Public License v3.0 |
| setup_pi.sh | ~5.8 KB | Automated Raspberry Pi Zero W environment setup script |
| test_crypto.py | ~10.8 KB | Full 3-phase ECDH + ASCON cryptographic handshake simulation |

Git History (3 commits):

Initial commit — repository creation with LICENSE

Correct header formatting in README.md

ECDH+ASCON Architecture — setup script and cryptographic handshake implementation

# 4. Environment Setup (setup_pi.sh)

A production-quality Bash setup script has been completed for provisioning the Raspberry Pi Zero W. Key design decisions include:

Preflight checks: Verifies root privileges, APT availability, and Python 3 installation before proceeding.

RAM-safe dependency strategy: Installs python3-cryptography from APT (pre-compiled ARM binary) instead of pip, avoiding the Rust compilation step that would exhaust the Pi Zero W's 512 MB RAM.

Virtual environment with system-site-packages: Creates ~/iot-env with --system-site-packages flag to inherit the system-installed cryptography library without recompilation.

Lightweight pip install: Only the ascon package (pure Python, <20 KB) is installed via pip inside the venv.

Automated verification: Runs an inline Python check to confirm that X25519, HKDF, and ASCON imports all succeed.

Coloured terminal output: Uses ANSI colour codes for clear INFO/WARN/ERROR messaging.

Strict error handling: Uses set -euo pipefail for robust failure detection.

# 5. Cryptographic Handshake Implementation (test_crypto.py)

The main cryptographic implementation is a 251-line Python script that simulates a complete IoT node-to-gateway handshake in three phases. The script is self-contained, well-documented, and designed to run on the Raspberry Pi Zero W within the provisioned virtual environment.

# 6. Phase 1 — Ephemeral X25519 ECDH Key Exchange

What has been implemented:

Both CLIENT and SERVER generate fresh ephemeral X25519 private keys (never persisted to disk).

Public keys are derived and exchanged (simulated in-process).

Each party independently computes the Diffie-Hellman shared secret via scalar multiplication.

An assertion verifies that both parties arrive at an identical 32-byte shared secret.

Verbose logging outputs public keys and shared secrets in hexadecimal for verification.

# 7. Phase 2 — HKDF-SHA256 Key Derivation

What has been implemented:

Derives a 16-byte (128-bit) symmetric key from the raw 32-byte ECDH shared secret.

Uses HKDF (RFC 5869) with SHA-256 as the hash algorithm.

Context binding via info=b"ascon-key" ensures the derived key is purpose-specific to the ASCON cipher.

No salt is used (acceptable when the input keying material has high entropy, as X25519 output does).

Both client and server derive keys independently; an assertion confirms they match.

# 8. Phase 3 — ASCON-128 AEAD Encrypt / Decrypt

What has been implemented:

Encrypts a simulated sensor payload (b"Sensor Data: 45.2C") using ASCON-128.

Uses a 16-byte random nonce (os.urandom) — unique per encryption to prevent nonce reuse attacks.

Associated data is set to empty (with a comment guiding production use of session-ID binding).

Decryption round-trip: the ciphertext is decrypted and verified against the original plaintext.

Tamper detection test: a single bit of the ciphertext is flipped, and the script verifies that ASCON correctly rejects the tampered ciphertext (decrypt returns None when authentication fails).

Output includes ciphertext length breakdown (plaintext length + 16-byte authentication tag).

# 9. Security Considerations Addressed

| Concern | How It Is Addressed |
| --- | --- |
| Ephemeral keys | Private keys are generated per-session and never written to disk, providing forward secrecy. |
| Key derivation | Raw ECDH output is never used directly as an encryption key; HKDF stretches it into a purpose-bound key. |
| Nonce uniqueness | Each encryption uses a fresh random nonce from os.urandom to prevent nonce-reuse attacks. |
| Authentication | ASCON-128 is an AEAD cipher — every ciphertext carries a 16-byte authentication tag that detects tampering. |
| Tamper rejection | Explicitly tested: flipped ciphertext bits cause authentication failure (decrypt returns None). |
| Constrained-device safety | The setup script avoids compiling Rust/C on the Pi Zero W, which would exhaust RAM. |
| UTF-8 robustness | The script reconfigures stdout to UTF-8 on Windows to avoid encoding crashes. |

# 10. Licensing

The project is released under the GNU General Public License v3.0 (GPLv3). The full license text is included in the repository as the LICENSE file.

# 11. What Remains / Next Steps

The following areas represent potential future work beyond what has been completed:

Multi-node networking: Implement actual socket-based or MQTT communication between multiple IoT nodes and a gateway.

Gateway server: Build a central gateway service that manages key exchange sessions with multiple client nodes concurrently.

Dynamic key rotation: Implement periodic re-keying to refresh session keys at defined intervals.

Hardware deployment: Deploy and test the setup on physical Raspberry Pi Zero W devices.

Performance benchmarking: Measure encryption/decryption throughput and latency on the constrained hardware.

Certificate-based authentication: Add device identity verification using pre-shared certificates or PSK.

Sensor data pipeline: Integrate real sensor readings (temperature, humidity, etc.) instead of simulated payloads.

Logging & monitoring: Add structured logging for production-grade auditability.

README expansion: Add architecture diagrams, usage instructions, and contribution guidelines.